ChE 467

Lecture 1 - Introductory Concepts

Stoichiometry

- Invented by Lavosier

- Molecules react in fixed proportions

Stoichiometric Coefficent, n

- Number of molecules produced when reaction goes once

Example

CO+12O2⇒CO2

Reaction rate:

Original definition due to Priestley - moles/hr produced by a reactor.

Van't Hoff showed that as you make a reactor bigger, you produce more product.  The production rate is proportional to the volume.  i.e.

total production(moles/hr)=constantVolume.

Van't Hoff defined

(1)

- rA called the Rate of production of A

- rA has dimensions moles/lit-hr

- rA is positive for a product, negative for a reactant

- Equ 1 applies to homogeneous reactions

- Different equation applies to heterogeneous reactions

- Some reactions scale as surface area instead

- RA is also called the Rate of production of A

- RA has dimensions moles/cm2-hr

- RA is positive for a product, negative for a reactant

- It is also useful to define ri the rate of a reaction i by

| - Table 2.1 Summary of the key definitions. |  |
| --- | --- |
| - Stoichiometric coefficient | - The amount of product produced when the reaction goes once. The stoichiometric is positive for a product and negative for a reactant. |
| - rA1 | - The net rate of production of a species A. rA is positive for a product and negative for a reactant. |
| - Rate of reaction 1 - r1 | - r1=1βArA for any species A participating in reaction1. |
| - Homogeneous reaction | - A reaction which happens throughout the reacting phase. |
| - Heterogeneous reaction | - A reaction which happens near the boundary of a reacting phase. |

- Variations in rate with conditions

- rates vary with:

- Concentrations of all species (reactants, products, inerts) (factors of 10-100)

- Temperature (factors of 100 or more)

- The presence of solvents (factors of 1012 or more)

- The presence of catalysts (factors of 1012 or more)

- rate equations

- definition:

- Rate as a function of the concentration of all of the speices in the reactor.

Typical rate laws for simple AC reactions:

rA= -k(CA)		First order

rA= -k(CA)2	Second order

rA= -k(CA)3	Third order

rA= -k(CA)n	nth order

n is the order

k is the rate constant

For reactions A + B  C

rA= -k(CA)n(CB)m

nth order in A, mth order in B

overall (m+n)th order

rA= -k(CA)(CB)2

first order in A, second order in B, third order overall.

| Table 2.4 The key definitions from Section 2.3. |  |
| --- | --- |
| Rate equation | The rate as a function of the concentration of the reactants. |
| Order | The exponent n is the expression. |
| First order reaction | A reaction whose rate is preparation to the reactant concentration to the first power (i.e. n = 1 in eqn. (2.11)). |
| Second order reaction | A reaction whose rate is proportional to the reactant concentration to the second order. |
| Overall order of reaction | The sum of the orders for each of the reactants. |

notation

k1, k2   rate constants

K1, K2 equilibrium constants

CA=[A] concentration of species A

Discussion Problems:

| Table2.2 Sample rate data to illustrate equation (2.11). |  |  |  |
| --- | --- | --- | --- |
| CA Moles/Liter | rate Moles/Lit/Min | CA Moles/Liter | rate Moles/Lit/hrs |
| 0.25 | 0.13 | 1 | 0.5 |
| 0.5 | 0.25 | 2 | 1.0 |

1. what is the order of the reaction?

| Table2.3 Sample data to illustrate equation (2.12). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CA moles/liter | CB moles/liter | rate moles/liter-min | CA moles/liter | CB moles/liter | rate moles/liter-min |
| 1 | 0.25 | 0.031 | 0.25 | 1 | 0.13 |
| 1 | 0.5 | 0.13 | 0.5 | 1 | 0.25 |
| 1 | 1 | .5 | 1 | 1 | 0.5 |
| 1 | 2 | 2.0 | 2 | 1 | 1.0 |

- 2. what is the order of the reaction?

More complex rate equations

Very few real reactions have simple reaction orders over a wide range of conditions:

Figure 2.1  The reproduction rate of paramecium as a function of the paramecium concentration and the rate of E. Coli growth in sugar solutions as a function of the sugar concentration.  Paramecium data of Meyers, H. Experim. Zoology 49 (1927) 1.  E. Coli data from Monod[1942].

recoli=k1K2E..ColiSugar(1+K2[Sugar])

(2.18)

- Figure 2.3 The rate of CH3NC isomerization to CH3CN as a function of the CH3NC pressure.

Figure2.15  The influence of the CO pressure on the rate of CO oxidation on Rh(111).  Data of Schwartz, Schmidt, and Fisher.

Called a Langmuir-Hinshelwood rate law.

rCO=k1PCOPO21+K2PCO2

also called Monod rate law

Arises because reaction occurs on catalyst surface:

Figure 12.33  The changes in A and B as a function of PA with KBPB=10.

- No industrially important reaction is first or second order over a wide range of conditions

Not all reactions have rate equations:

Figure2.22  Rate Data for CO Oxidation on Rh(100) catalyst.  Data of Schwartz et al.[1986].

Summary

| - Table 2.1 Summary of the key definitions. |  |
| --- | --- |
| - Stoichiometric coefficient | - The amount of product produced when the reaction goes once. The stoichiometric is positive for a product and negative for a reactant. |
| - rA1 | - The net rate of production of a species A. rA is positive for a product and negative for a reactant. |
| - Rate of reaction 1 - r1 | - r1=1βArA for any species A participating in reaction1. |
| - Homogeneous reaction | - A reaction which happens throughout the reacting phase. |
| - Heterogeneous reaction | - A reaction which happens near the boundary of a reacting phase. |

| Table 2.4 The key definitions from Section 2.3. |  |
| --- | --- |
| Rate equation | The rate as a function of the concentration of the reactants. |
| Order | The exponent n is the expression. |
| First order reaction | A reaction whose rate is preparation to the reactant concentration to the first power (i.e. n = 1 in eqn. (2.11)). |
| Second order reaction | A reaction whose rate is proportional to the reactant concentration to the second order. |
| Overall order of reaction | The sum of the orders for each of the reactants. |

- Real reactions rarely follow these simple rate laws.

- Some reactions do not have a rate law

Temperature dependence:

Harcourt Equation (assumes energy transfer dominates):

k=kT Tn

k=rate constant

kT=preexponential

n = constant between 1 and 4.

Arrhenius' model (assumes barrier)

k=koexp(-Ea/kBT)

ko=preexponential

Ea = activation barrier, kj/molecule

k = boltzman's constant, 1.381x10-23 j/K

T= temp (kelvin).

Real data somewhere in between

Figure 2.6  The rate of the reaction CH + N2 HCN + N as a function of the temperature.  Data of Becker, Gelger and Wresen[1996].

k1=km0(T)me-EA±BT

(2.28)

Arrhenius’ effect much larger than Harcourt and Essen.

Key implications of Arrhenius’ Law

Figure 2.8  A plot of ½ vs. EA at 100, 200, 300, 400, and 500 K.

EA=1/15 kcal/mole-∘KTminute

(2.31)

EA=0.06 kcal/mole-∘KTsec

(2.32)

Tminute=15 K-molekcalEA

(2.33)

Changes in rate with temp

r2=r1expEAB1T1−1T2

(2.36)

Figure 2.4  The fractional change in the rate of a nth order reaction when the temperature is changed from 25 to 35 C.

| Table 2.6 The variation in rate of a series of reactions with a 10o K change in temperature. Data from Van’t Hoff[1884]. |  |  |
| --- | --- | --- |
| Reaction | Temp. range C | Rate change with a 10 K temperature change |
| CH3COOCH2CH3+H2O⇒H+CH3COOH+CH3CH2OH | 3.6-30.4 | 2.03 |
| CH3CH2Cl+NaOH⇒H2C=CH2+NaCl+H2O | 23.5-43.6 | 2.87 |
| CH3CH2CH2Cl+NaOH⇒CH3CH=CH2+NaCl | 24.5-43.6 | 2.68 |
| HPO3+H2O⇒H3PO4 | 0-61 | 3.0 |

| Table 2.7 The variation in the respiration rate of plants with a 10 change in temperature. Data of Clausen[1890]. |  |
| --- | --- |
| Wheat | 2.47 |
| Lilac | 2.48 |
| Lupine | 2.46 |

Figure 2.10 The rate of E. Coli growth as a function of temperature adapted from Bailey and Ollis [1977].

Figure 2.11  The rate that crickets chirp as a function of temperature.  Data for field crickets (Gryllys pennsylvanicus)

Again: not all reactions work:

Figure 2.17  The variation in the rate of the reaction in Figure 2.16 with temperature.  Data of Loffler and Schmidt[1976].  A) PNH3= 0.3, PH2=0.15, B)  PNH3= 0.3, PH2=0.44, C) PNH3= 0.05, PH2=0.15, D) PNH3= 0.05, PH2=0.45

Discussion Problem

Your taste buds work by a chemical reaction where sugar molecules bind to nerve endings in your mouth.

- What is the activation barrier for the process?

- How much will the sweetness of bread change if you heat the bread enough that the temperature of your tongue rises by 5C?