- ChE 467 Overview of chapter 3 - Analysis of rate data

- Rate measurements and old topic

- General approach

- Initiate reaction

measure concentration vs time

fit data to calculate rates

Figure 3.2  Wilhelmy’s [1850] measurements of the changes in sucrose concentration in grape juice after acid is added.

Figure 3.3 Concentration vs time for a simple reaction.

These types of measurements started in 1820.  Still done today.  (now do them faster).

| Table 3.1 Some techniques used to measure rates of reaction. |  |  |
| --- | --- | --- |
| Method | Description | Time Scale |
|  | Batch Methods |  |
| Conventional | - Mix reactants together in a batch reactor - Measure concentration versus time | 10 sec or more |
| Stopped flow | - Set of continuous flow systems where reactants are fed into the reactor, and flow out again so quickly that there is negligible reaction - Stop the flow so the reactants can react - Measure conversion versus time | 10-1 sec or more |
| Temperature jump | - Mix reactants at such a low temperature that the reaction rate is negligible - Use CO2 laser to suddenly heat reaction - Measure concentration vs time | 10-6 sec or more |
| Shock tube | - Put 10-1 atm of one reactant and 10 atm at helium on one side of a diaphragm - Put 10-3 atm of the other reactant on the other side of the diaphragm - Suddenly break the diaphragm so the gas flows from the high pressure side to the low pressure side Measure the reactant concentration vs time | 10-3 to 10-5 sec |
| Flash photolysis | - Put the reactants into a vessel under conditions where reaction is negligible - Pulse a laser or flash lamp to start reaction - Measure the reactant concentration vs time | 10-9 to 10-1 sec |
| NMR | - Initiate a change with a magnetic pulse - Measure the decay of spins with the NMR | 10-2 to 10-9 sec |
| Flow Methods |  |  |
| Conventional flow system | - Continuously feed reactants into a reactor - CSTR or plug flow - Measure the steady state reaction rate | 10-3 sec or more |
| Molecular beam | - Direct beams of reactants toward each together in a vacuum system - Measure the steady state reaction rate | 10-13 to 10-9 sec |

If you have to do an experiment which do you choose?

- Direct vs indirect methods

- Choose method with appropriate time scale

- -----

- Direct vs indirect methods

- recall - rate equation is the rate as a function of the concentrations

- Direct method - any method where you actually measure the rate as a function of concentration

- Indirect method - a method where you measure some other property (i.e. concentration vs time) and infer a rate equation.

- Example of a direct method:

- Figure 3.7  A typical arsine decomposition reactor.

- 2AsH3⇒2As+3H2

- (3.6)

- Figure 3.8  A possible apparatus to examine the decomposition of arsine (AsH3) on silicon.

- PAsH3=PAsH30´e−k1t

- (3.8)

- Indirect measurement to do the same thing

- Figure 3.9  Typical batch data for reaction(3.7).  Data of Tamaru[1955].

| - Table 3.2 A Comparison of the Advantages and Disadvantages of Direct and Indirect Methods |  |
| --- | --- |
| - Direct Method | - Indirect Method |
| - Advantages - Get rate equation directly - Easy to fit data to a rate law - High confidence on final rate equation | - Disadvantages - Must infer rate equation - Hard to analyze rate data - Low confidence on final rate equation |
| - Disadvantages - Difficult experiment - Need many runs - Not suitable for very fast or very slow reactions | - Advantages - Easier experiment - Can do a few runs and get important information - Suitable for all reactions including very fast or very slow ones |

- Other notation

- direct method - differential method

- -differential reactor

- indirect method - integral method

- Next: start analysis of data from indirect reactors:

- Which is easier to analyze?

- Direct method (rate vs concentration)

- Indirect method (concentration vs time)

- Direct is easier to analyze.

Analysis of data from a differential reactor

- Figure 3.11  The rate of copper etching as a function of the oxygen concentration.  Data of Steger and Masel [1998].

- General method - least squares with rate vs time data.

- Pitfalls

- It is not uncommon for more than one rate equation may fit the measured kinetics within the experimental uncertainties, just because data fits, does not mean rate equation is correct.

- The quality of kinetic data vary with the equipment used and the method of temperature measurement and control.  Data taken on one apparatus is often not directly comparable to data taken on different apparatus.

- It is not uncommon to observe 10-30% variations in rate taken in the same apparatus on different days.  Usually, these variations can be traced to variations in the temperature, pressure, or flow rate in the reactor.

- The procedure used to fit the data can have a major effect on the values of the parameters obtained in the data analysis.

- The quality of the regression coefficient (r2) does not tell you how well a model fits your data.

- Example 3.A Fitting data to Monod's Law

Table 3.A.1 shows some data for the growth rate of paramecium as a function of the paramecium concentration.  Fit the data to Monod's Law:

rp=k1K2[par]1+K2[par]

(3.A.1)

where [par] is the paramecium concentration, and k1 and K2 are constants.

| Table 3.A.1 The rate of paramecium reproduction as a function of the paramecium concentration. Data of Meyers, J. Experimental Zoology, 49 (1927) 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Paramecium concentration #/CC | rate, #/CC-hr | Paramecium concentration #/CC | rate, #/CC-hr | Paramecium concentration #/CC | rate, #/CC-hr |
| 2 | 10.4 | 16 | 36 | 46 | 96 |
| 3.6 | 12.8 | 16.6 | 46.4 | 46.2 | 124.8 |
| 4 | 23.2 | 19 | 59.2 | 47.4 | 117.6 |
| 5.2 | 17.6 | 20 | 62.4 | 55 | 112 |
| 7.8 | 46.4 | 23.8 | 62.4 | 57 | 127.2 |
| 8 | 23.2 | 26 | 57.6 | 61 | 116 |
| 8 | 46.4 | 30.4 | 108.8 | 61.6 | 111.2 |
| 11 | 32 | 31 | 80 | 71 | 124 |
| 14.4 | 34.4 | 31.2 | 61.6 | 74 | 116 |
| 15.6 | 44.8 | 31.6 | 109.6 | 76.4 | 116 |
| 15.6 | 63.2 | 39.2 | 103.2 |  |  |

There are two methods that people use to solve problems like this:

- Rearranging the equations to get a linear fit and using least squares

- Doing non-linear least squares

I prefer the latter, but I wanted to give a picture of the former.

There are two versions of the linear plots:

- Lineweaver-Burk Plots

- Eadie-Hofstee Plots

In the Lineweaver-Burk method, one plots 1/rate vs. 1/concentration.  Rearranging equation (3.A.1) shows:

1rp=1k1K2[par]+1k1

(3.A.2)

| Figure 3.A.1 A Lineweaver-Burk plot of the data in Table 3.A.1 | Figure 3.A.2 The Lineweaver-Burk fit of the data in Table 3.A.1 |
| --- | --- |

| Table 3.A.2 The formulas in the spreadsheet for the Lineweaver Burke plot |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F |
| 01 |  | k_1 | =1/D2 | =SLOPE(D6:D35,C6:C35) |  | 2 |
| 02 |  | K_2 | =1/C1/D1 | =INTERCEPT(D6:D36,C6:C36) |  | =SUM(F5:F37) |
| 03 |  |  | r2 | =RSQ(D6:D36,C6:C36) |  |  |
| 04 | conc | rate | 1/conc | 1/rate | rate calculated from rate equation | error |
| 05 | 0 | 0 |  |  | =C$1*C$2*$A5/(1+C$2*$A5) | =ABS(E5-$B5)^$F$1 |
| 06 | 2 | 10.4 | =1/A6 | =1/B6 | =C$1*C$2*$A6/(1+C$2*$A6) | =ABS(E6-$B6)^$F$1 |
| 07 | 3.6 | 12.8 | =1/A7 | =1/B7 | =C$1*C$2*$A7/(1+C$2*$A7) | =ABS(E7-$B7)^$F$1 |

Comparison of equations (3.A.2) and (3.A.3) shows:

k1 = 1/.00717=139.4, k2=1/(0.194*k1)=0.037,

r2=0.930

| Figure 3.A.1 A Lineweaver-Burk plot of the data in Table 3.A.1 | Figure 3.A.2 The Lineweaver-Burk fit of the data in Table 3.A.1 |
| --- | --- |

The Eadie-Hofstee plot

Rearranging equation (3.A.1):

rp1+K2[par]=k1K2[par]

(3.A.4)

Further rearrangement yields:

rp[par]=k1K2−K2rp

(3.A.5)

| Figure 3.A.3 An Eadie-Hofstee plot of the data in Table 3.A.1 | Figure 3.A.4 The Eadie-Hofstee fit of the data in Table 3.A.1 |
| --- | --- |

rp[par]=4.18−0.0156rp

(3.A.6)

r2=0.34

| Figure 3.A.3 An Eadie-Hofstee plot of the data in Table 3.A.1 | Figure 3.A.4 The Eadie-Hofstee fit of the data in Table 3.A.1 |
| --- | --- |

Eadie-Hofstee gives much lower r2 but better fit to data!

The last way to fit the data is with a non-linear least squares.

The idea in nonlinear least squares is to use the solver function of a spreadsheet to calculate the best values of the coefficients based on some criterion.   A common criterion is to minimize the total error, where the total error is defined by:

TotalError=Dataabsrp−k1K2[par]1+K2[par]2

(3.A.7)

One often uses powers other than 2 to do the fitting.

| Figure 3.A.5 A nonlinear least squares fit to the data in Table 3.A.1 | Figure 3.A.6 A comparison of the three fits to the data |
| --- | --- |

| Table 3.A.5 A comparison of the various fits to the data in Table 3.A.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Method | k1 | K2 | Total error | R-squared |
| Lineweaver-Burk | 139 | 0.0370 | 9643 | 0.910 (linear plot) |
| Eadie-Hofstee | 267 | 0.0156 | 6809 | 0.344 (linear plot) |
| non-linear least squares | 204 | 0.0221 | 4919 | 0.905 (non-linear) |

| Table 3.A.7 The values of R-squared calculated using the different methods |  |  |  |
| --- | --- | --- | --- |
| Method | R-squared from linear regression | R-squared from equation (2.B.9) | R-squared from equation (2.B.10) |
| Lineweaver-Burk | 0.910 | 0.814 | 0.552 |
| Eadie-Hofstee | 0.344 | 0.869 | 0.516 |
| nonlinear least squares | 0.905 | 0.905 | 0.558 |

- Example 3.B -  Tests of Statistical Significance:  Analysis of Variance

| Table 3.B.1 Fits to the data in Example 3.A |  |  |  |  |
| --- | --- | --- | --- | --- |
| ### Concentration | Experimental Rate | Calculated rate Non-linear least squares | Calculated rate Lineweaver-Burk | Calculated rate Eadie-Hofstee |
| 0 2 3.6 4 5.2 7.8 8 8 | 0 10.4 12.8 23.2 17.6 46.4 32 34.4 | 0 8.65 15.06 16.60 21.07 30.05 30.71 30.71 | 0 9.6 16.38 17.69 22.49 31.21 31.83 31.83 | 0 8.10 14.24 15.73 20.10 29.06 29.72 29.72 |
| Continued |  |  |  |  |
| Variance |  | 164 | 230 | 321 |

- Which model fits best?  Is the difference statistically significant?

First let us see which model fits best.  We do that by calculating the variance of the data and seeing which model has the lowest variance.  The variance Vi is defined by

Vi=pointsexperimentalrate−calculatedrate2numberofsamples−numberofindependentparametersinmodel

(3.B.1)

substituting in equation (3.A.7) yields

Vi=total error from Equ. 3.A.7numberofsamples-numberofparameters

(3.B.2)

It is important to calculate the variance as shown in (3.B.1) and not for example the variance of one over the rate.  In order to use the statistical tests below, one will have to assume that the error in the data follows what statisticians call a “2 distribution.”  If you calculate the errors in the rate, the errors usually do follow a “2 distribution”.

I used Excel to calculate

1-FDIST (1.96, 30, 30) = 0.97

so I am 97% sure that the non-linear least squares fit better than the Lineweaver-Burk plot.  Excel also has a FINV function that calculates Finverse via

#### Analysis of batch reactor data

CAfCAodCA−rA

(3.31)

1n−1knCAon−1CAoCAfn-1−1=τ

1k1LnCAoCAf=τ

Typical batch reactor data looks like

Two methods to analyze data

Essen's method

Van't Hoffs Method

Essen's method:

- Essen's Method

- Figure 3.15  A replot of the data from Figure 3.14 as a function of ln CA/CA0and CA0CA..

Never works in practice

- Figure 3.16  An Essen plot of the data in Table 3.5.

Van't hoff's method

Calculate k  - is it constant

(3.51)

1n−1knCA0n−1CA0CAn-1−1=t

Figure 3.18  Van’t Hoff Plot of the data from Table 3.5 and Table 3.6

Another interesting thing in the chapter

Table 3.D.3 Module used to calculate k1, k2, k3, where k1, k2 and k3 are defined by equation (3.D.2)

Public Function kone(ca0, ca, tau) As Variant kone = Log(ca0 / ca) / tau End Function Public Function ktwo(ca0, ca, tau) As Variant ktwo = ((1# / ca) - (1# / ca0)) / tau End Function Public Function kthree(ca0, ca, tau) As Variant kthree = ((1# / ca) ^ 2 - (1# / ca0) ^ 2) / tau End Function

| Table 3.1 Summary of key concepts. |  |
| --- | --- |
| Two methods to measure rates: Direct and Indirect |  |
| Direct | Indirect |
| High Accuracy Need many runs | Lower Accuracy Fewer experiments |
| Methods to analyze direct data - Least squares - Non-linear least squares Non-linear least squares easier and more accurate |  |
| Methods to analyze indirect data Essen - construct plots of LnCAOCA,CAOCAn−1 - see if linear Van’t Hoff - calc K1, K2, K3 - see if constant Van’t Hoff - easier and more accurate |  |
| Key equations for indirect data NA00XAdXAV−rA=t (3.26) CA00XAdXA−rA=t (3.28) òCAFCA0dCA−rA (3.31) 1k1LnCA0CA=t (3.39) 1n−1knCA0n−1CA0CAn-1−1=t (3.42) |  |