;twosumlasmfloat.sa  Sum of products. Separate accum of even/odd terms

;Using double-word load LDDW for floating-point implementation

loop:		LDDW		*aptr++, ai1:ai0	;64-bit word ai0 and ai1

LDDW		*bptr++, bi1:bi0	;64-bit word  bi0 and bi1

MPYSP		ai0, bi0, prodl	;lower 32-bit product

MPYSP		ai1, bi1, prodh	;higher 32-bit product

ADDSP		prodl, suml, suml	;accum 32-bit even terms

ADDSP		prodh, sumh, sumh	;accum 32-bit odd terms

SUB		count, 1, count	;decrement count

[count]	B		loop			;branch to loop