;dotppfloat.asm  With parallel instructions - Floating-point

MVK   .S1	200, A1	;count into A1

||		ZERO	.L1	A7		;init A7 for accum

LOOP		LDW	.D1	*A4++,A2	;A2=32-bit data pointed by A4

||		LDW	.D2   *B4++,B2	;B2=32-bit data pointed by B4

SUB	.S1	A1,1,A1	;decrement count

NOP		2		;delay slots for both LDW and B

[A1]	B	.S2 	LOOP		;branch to LOOP (after ADDSP)

MPYSP .M1x  A2,B2,A6	;product in A6

NOP	  	3		;3 delay slots for MPYSP

ADDSP	.L1	A6,A7,A7	;accum in A7,then branch

;branch occurs here