;twosumfloat.asm For two sums of products - Floating-point

MVK	.S1	100, A1	;count/2 into A1

||		ZERO	.L1	A7		;init A7 for accum of even terms

||		ZERO	.L2	B7		;init B7 for accum of odd terms

LOOP		LDDW	.D1  	*A4++,A3:A2	;64-bit into register pair A2,A3

||		LDDW	.D2 	*B4++,B3:B2	;64-bit into register pair B2,B3

SUB	.S1	A1,1,A1	;decrement count

NOP	   	2		;delay slots for LDW

[A1]	B	.S2	LOOP		;branch to LOOP

MPYSP .M1x	A2,B2,A6	;lower 32-bit product in A6

||		MPYSP	.M2x	B3,A3,B6	;upper 32-bit product in B6

NOP		3		;3 delay slot for MPYSP

ADDSP	.L1	A6,A7,A7	;accum even terms in A7

||		ADDSP	.L2	B6,B7,B7	;accum odd terms in B7

;branch occurs here

NOP		3		;delay slots for last ADDSP

ADDSP	.L1x	A7,B7,A4	;final sum of even and odd terms

NOP		3		;delay slots for ADDSP