;dotpnpfloat.asm  ASM with no parallel instructions for floating-point

MVK	.S1	200, A1	;count into A1

ZERO	.L1	A7		;init A7 for accum

LOOP		LDW	.D1	*A4++,A2	;A2=32-bit data pointed by A4			LDW	.D1 	*A8++,A3	;A3=32-bit data pointed by A8

NOP		4		;4 delay slots for LDW

MPYSP .M1  	A2,A3,A6	;product in A6

NOP		3		;3 delay slots for MPYSP

ADDSP .L1	A6,A7,A7	;accum in A7

SUB	.S1	A1,1,A1	;decrement count

[A1]	B    	.S2 	LOOP		;branch to LOOP