;dotpipedfix.asm  ASM code for dot product with software pipelining

;For fixed-point implementation

;cycle 1

MVK	.S1	100,A1	;loop count

||  	ZERO	.L1	A7		;init accum A7

||		ZERO	.L2	B7		;init accum B7

||		LDW	.D1	*A4++,A2	;32-bit data in A2

||		LDW	.D2	*B4++,B2	;32-bit data in B2

;cycle 2

||		LDW	.D1	*A4++,A2	;32-bit data in A2

||		LDW	.D2	*B4++,B2	;32-bit data in B2

||	[A1]	SUB	.S1	A1,1,A1	;decrement count

;cycle 3

||		LDW	.D1	*A4++,A2	;32-bit data in A2

||		LDW	.D2	*B4++,B2	;32-bit data in B2

|| [A1]	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

;cycle 4

||		LDW	.D1	*A4++,A2	;32-bit data in A2

||		LDW	.D2	*B4++,B2	;32-bit data in B2

|| [A1] 	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

;cycle 5

||		LDW	.D1	*A4++,A2	;32-bit data in A2

||		LDW	.D2	*B4++,B2	;32-bit data in B2

|| [A1]	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

;cycle 6

||		LDW	.D1	*A4++,A2	;32-bit data in A2

||		LDW	.D2	*B4++,B2	;32-bit data in B2

|| [A1]	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

||		MPY	.M1x	A2,B2,A6	;lower 16-bit product into A6

||		MPYH	.M2x	B2,A2,B6	;upper 16-bit product into B6

;cycle 7

||		LDW	.D1	*A4++,A2	;32-bit data in A2

||		LDW	.D2	*B4++,B2	;32-bit data in B2

|| [A1]	SUB	.S1	A1,1,A1	;decrement count

||	[A1]	B	.S2	LOOP		;branch to LOOP

||		MPY	.M1x	A2,B2,A6	;lower 16-bit product into A6

||		MPYH	.M2x	B2,A2,B6	;upper 16-bit product into B6

;cycles 8-107 (loop cycle)

||		LDW	.D1	*A4++,A2	;32-bit data in A2

||		LDW	.D2	*B4++,B2	;32-bit data in B2

||	[A1]	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

||		MPY	.M1x	A2,B2,A6	;lower 16-bit product into A6

||		MPYH	.M2x	B2,A2,B6	;upper 16-bit product into B6

||		ADD	.L1	A6,A7,A7	;accum in A7

||		ADD	.L2	B6,B7,B7	;accum in B7

;branch occurs here

;cycle 108 (epilog)

ADD	.L1x	A7,B7,A4	;final accum of odd/even