# OptSim™ Application Note

## FTTH System with Broadband PON Access Architecture

RSoft Design Group, Inc.

200 Executive Blvd.

Ossining, NY 10562

info@rsoftdesign.com

www.rsoftdesign.com

Copyright © 2005 by RSoft Design Group, Inc.

All rights reserved.

## Example: FTTH System with Broadband PON access architecture.

Design and deployment activities for FTTH (fiber-to-the-home) and FTTP (fiber-to-the-premises) access networks are on the rise and rapidly growing in order to support the increasing demands and delivery of new multimedia services to the customer premises such as interactive video, voice, and high-speed internet.

There are many types of FTTH technologies; the most popular one is based on the concept of using a passive fiber distribution network, known as a passive optical network (PON). FTTH employing PON access architecture is the accepted choice of delivery channel for triple-play services (voice, video and data) from service providers to the home and business users.  Three major PON technologies are currently under consideration as the basis for FTTH deployments: Broadband PON (BPON), Gigabit PON (GPON), and Ethernet PON (EPON).

Broadband PON is the most mature and widely used among them to the date. BPON is a set of standards that specify the service capabilities and network protocols for broadband services over fiber access. It is specified by the ITU-T, and published in the G.983.x series of ITU-T recommendations.

In a PON, the active optoelectronics are situated on either ends of the passive network. An optical line termination (OLT) device is installed in the central office (CO), and an optical network termination (ONT) device is installed on the other end, in or near each home or business site. Fiber distribution is done using a tree-and-branch architecture. A single fiber connected to the OLT can be split up to 32 times and connected to multiple ONTs.

Figure 1.  Layout for FTTH BPON system.

Current simulation example models a typical BPON FTTH design with 32 subscribers and 20-km reach.  Figure 1 depicts the OptSim schematic for example layout. The Central Office is connected through a 15-km standard single-mode fiber to the first Remote Node with a 1:4 splitter. Each of the four outputs goes through another 4.5-km fiber and then enters the Remote Node with a 1:8 splitter. Outputs from the 1:8 splitter are connected to eight end-users at the ONT though drop-off cables of length varying from 100 to 900 feet.

Figure 2. Output spectrum from Central Office.

Figure 3. Signal constellation (left) and 4-level logical signal (right) at 16-QAM encoder.

The triple-play service is realized as a combination of data, voice, and video signals. To optimize the bandwidth in BPON the transmission through the optical fiber path employs the CWDM technique with data and voice component transmitted at wavelengths in the range of 1480-1500 nm, and video within the 1550-1560 nm range. The high-speed internet component is represented by a data link with 1.25 Gb/s downstream bandwidth. The voice component can be represented as VOIP service (voice over IP, packet-switched protocol), which is gaining popularity as an alternative to traditional PSTN (public switched telephone network) with POTS (plain old telephone service) at the customer end. The video component is represented as a 16-QAM subcarrier multiplexed (SCM) system. Actually, 64-QAM is more typical modulation scheme used to carry digital video/data stream in RF channels but we used 16-QAM for simplicity. Also for simplicity purpose this simulation example models only a downstream link with one ONT unit attached. As an exercise the reader can add more ONT units and re-run the simulations.

Figure 4. Received eye diagram for data signal

Figure 5. Received eye diagram for data signal

Figure 2 shows the signal spectrum output from OLT with data/voice signal at 1500 nm and video signal at 1550 nm. Figure 3 demonstrates 4-level logical signal and signal constellation plot for 16-QAM encoder.

The signal from the central office travels though a 20-km long fiber distribution network and arrives at optical network termination unit. The optical spectrum at the input to ONT is shown at Figure 4. Here the optical signal first demultiplexed into data/voice and video components. The data component goes to the optical receiver, PIN. Optical power meters inserted after transmitters and before receivers shows that total attenuation from fiber spans and splitters is about 29 dB and input power to the receiver is about –25 dBm. The receiver eye diagram for data signal is given at Figure 5. The video component of the received signal enters 16-QAM decoder. Figure 6 shows 4-level logical signal and signal constellation plot at the received side of 16-QAM decoder.  Figure 7 shows multi-level eye-pattern and electrical signal RF waveform at 16-QAM decoder.

Figure 6. Signal constellation (left) and 4-level logical signal (right) at 16-QAM encoder.

Figure 7. Eye-pattern (left) and output electrical signal (right) at 16-QAM encoder.

In conclusion, the given example allows user to study various performance characteristics and its dependence of components characteristics and layout’s specifics in physical layer design for BPON FTTH network.