Appendix A - Table of Metallic Element Crystal Structures

Data source: W. Steurer, in Physical Metallurgy (R.W. Cahn and P. Haasen, eds.), 4th edition, Elsevier, Amsterdam, pp. 17-46 (1996).

|  | Temperature (K) | Pressure (GPa) | Pearson Notation | Prototype Structure | c/a Ratio |
| --- | --- | --- | --- | --- | --- |
| -Li3 | <70 |  | hR3 | Sm |  |
| -Li3 |  |  | cI2 | W |  |
| -Li3 |  | >6.9 | cF4 | Cu |  |
| - Be4 |  |  | hP2 | Mg | 1.568 |
| - Be4 | >1543 |  | cI2 | W |  |
| - Be4 |  | >28.3 | hP8? |  | 0.789 |
| -Na11 | <40 |  | hR3 | Sm |  |
| - Na11 |  |  | cI2 | W |  |
| - Mg12 |  |  | hP2 | Mg | 1.624 |
| - Mg12 |  | >50 | cI2 | W |  |
| Al13 |  |  | cF4 | Cu |  |
| - Si14 |  |  | cF8 | C-diamond |  |
| - Si14 |  | >12 | tI4 | - Sn | 0.552 |
| - Si14 |  | >13.2 | hP1 | BiIn | 0.92 |
| - Si14 |  | >36 | o? |  |  |
| - Si14 |  | >43 | hP2 | Mg | 1.699 |
| - Si14 |  | >78 | cF4 | Cu |  |
| -K19 |  |  | cI2 | W |  |
| - K19 |  | >12 | cF4 | Cu |  |
| -Ca20 |  |  | cF4 | Cu |  |
| -Ca20 | >728 | or >19.5 | cI2 | W |  |
| -Ca20 |  | >32 | cP1 | -Po |  |
| - Sc21 |  |  | hP2 | Mg | 1.592 |
| - Sc21 | >1610 |  | cI2 | W |  |
| - Sc21 |  | >19 | tP4? |  |  |
| - Ti22 |  |  | hP2 | Mg | 1.587 |
| - Ti22 | >1155 |  | cI2 | W |  |
| - Ti22 |  | >2 | hP3 | - Ti |  |
| V23 |  |  | cI2 | W |  |
| Cr24 |  |  | cI2 | W |  |
| - Mn25 |  |  | cI58 | - Mn |  |
| - Mn25 | >1000 |  | cP20 | - Mn |  |
| - Mn25 | >1373 |  | cF4 | Cu |  |
| - Mn25 | >1411 |  | cI2 | W |  |
| - Fe26 |  |  | cI2 | W |  |
| - Fe26 | >1185 |  | cF4 | Cu |  |
| - Fe26 | >1667 |  | cI2 | W |  |
| - Fe26 |  | >13 | hP2 | Mg | 1.603 |
|  Co27 |  |  | hP2 | Mg | 1.623 |
| - Co27 | >695 |  | cF4 | Cu |  |
| Ni28 |  |  | cF4 | Cu |  |
| Cu29 |  |  | cF4 | Cu |  |
| Zn30 |  |  | hP2 | Mg | 1.856 |
| - Ga31 |  |  | oC8? | - Ga |  |
| - Ga31 | <330 | >1.2 | CI12 | Cu |  |
| - Ga31 | >330 | >3.0 | tI2 | In | 1.588 |
| - Ge32 |  |  | cF8 | C-diamond |  |
| - Ge32 |  | >11 | tI4 | - Sn | 0.551 |
| - Ge32 |  | >75 | hP1 | BiIn | 0.92 |
| - Ge32 |  | >106 | hP4 |  |  |
| - As33 |  |  | hR2 | - As | 2.805 |
| - As33 |  | >25.0 | cP1 | - Po |  |
| - Rb37 |  |  | cI2 | W |  |
| - Rb37 |  | >7.0 | cF4 | Cu |  |
| - Rb37 |  | >14 |  |  |  |
| - Rb37 |  | >17 |  |  |  |
| - Rb37 |  | >20 | tI4 |  |  |
| - Sr38 |  |  | cF4 | Cu |  |
| - Sr38 | >504 |  | hP2 | Mg | 1.636 |
| - Sr38 | >896 | or >3.5 | cI2 | W |  |
| - Sr38 |  | >26 |  |  |  |
| - Sr38 |  | >35 |  |  |  |
| - Y39 |  |  | hP2 | Mg | 1.571 |
| - Y39 | >1751 |  | cI2 | W |  |
| - Y39 |  | >10 | hR3 | Sm |  |
| - Y39 |  | >26 | hP4? |  |  |
| - Y39 |  | >39 | cF4 | Cu |  |
| - Zr40 |  |  | hP2 | Mg | 1.593 |
| - Zr40 | >1136 |  | cI2 | W |  |
| - Zr40 |  | >2 | hP3 | - Ti |  |
| - Zr40 |  | >30 | cI2 | W |  |
| Nb41 |  |  | cI2 | W |  |
| Mo42 |  |  | cI2 | W |  |
| Tc43 |  |  | hP2 | Mg | 1.604 |
| Ru44 |  |  | hP2 | Mg | 1.582 |
| Rh45 |  |  | cF4 | Cu |  |
| Pd46 |  |  | cF4 | Cu |  |
| Ag47 |  |  | cF4 | Cu |  |
| Cd48 |  |  | hP2 | Mg | 1.886 |
| In49 |  |  | tI2 | In | 1.5218 |
| - Sn50 | <291 |  | cF8 | C-diamond |  |
| - Sn50 | >291 |  | tI4 | - Sn | 0.546 |
| - Sn50 |  | >9.2 | tI2 | Pa | 0.91 |
| - Sn50 |  | >40 | cI2 | W |  |
| - Sb51 |  |  | hR2 | - As | 2.617 |
| - Sb51 |  | >8 | mP4 | - Sb |  |
| - Sb51 |  | >28 | cI2 | W |  |
| - Cs55 |  |  | cI2 | W |  |
| - Cs55 |  | >2.37 | cF4 | Cu |  |
| '- Cs55 |  | >4.22 | cF4 | Cu |  |
| - Cs55 |  | >4.27 | tI4 |  |  |
| - Cs55 |  | >10 |  |  |  |
| - Cs55 |  | >72 | cF4? |  |  |
| - Ba56 |  |  | cI2 | W |  |
| - Ba56 |  | >5.33 | hP2 | Mg | 1.581 |
| - Ba56 |  | >7.5 |  |  |  |
| - Ba56 |  | >12.6 |  |  |  |
| - La57 |  |  | hP4 | -La | 2 x 1.613 |
| - La57 | >583 | or >2.3 | cF4 | Cu |  |
| - La57 | >1138 |  | cI2 | W |  |
| - La57 |  | >7.0 | hP6 |  |  |
| - Ce58 | <96 |  | cF4 | Cu |  |
| - Ce58 |  |  | hP4 | -La | 2 x 1.611 |
| - Ce58 | >326 |  | cF4 | Cu |  |
| - Ce58 | >999 |  | cI2 | W |  |
| '- Ce58 |  | >5.1 | oC4 | -U? |  |
| - Ce58 |  | >12.2 | tI2 | In |  |
| - Pr59 |  |  | hP4 | -La | 2 x 1.611 |
| - Pr59 | >1068 |  | cI2 | W |  |
| - Pr59 |  | >3.8 | cF4 | Cu |  |
| - Pr59 |  | >6.2 | hP6 | Pr | 3 x 1.622 |
| - Pr59 |  | >20 | oC4 | -U |  |
| - Nd60 |  |  | hP4 | -La | 2 x 1.612 |
| - Nd60 | >1136 |  | cI2 | W |  |
| - Nd60 |  | >5.8 | cF4 | Cu |  |
| - Nd60 |  | >18 | hP6 | Pr | 3 x 1.611 |
| - Nd60 |  | >38 | mC4 |  |  |
| - Pm61 |  |  | hP4 | -La | 2 x 1.60 |
| - Pm61 | >1163 |  | cI2 | W |  |
| - Pm61 |  | >10 | cF4 | Cu |  |
| - Pm61 |  | >18 | hP6 | Pr |  |
| - Pm61 |  | >40 | ? |  |  |
| - Sm62 |  |  | hR3 | -Sm | 4.5 x 1.605 |
| - Sm62 | >1007 |  | hP2 | Mg | 1.596 |
| - Sm62 | >1195 |  | cI2 | W |  |
| - Sm62 |  | >4.5 | hP4 | -La | 2 x 1.611 |
| - Sm62 |  | >14 | cF4 | Cu |  |
| - Sm62 |  | >19 | hP6 | Pr | 3 x1.611 |
| - Sm62 |  | >33 | mC4 | ? |  |
| - Eu63 |  |  | cI2 | W |  |
| - Eu63 |  | >12.5 | hP2 | Mg | 1.553 |
| - Eu63 |  | >18 | ? |  |  |
| - Gd64 |  |  | hP2 | Mg | 1.591 |
| - Gd64 | >1508 |  | cI2 | W |  |
| - Gd64 |  | >2.0 | hR3 | -Sm | 4.5 x 1.61 |
| - Gd64 |  | >5 | hP4 | -La | 2 x 1.624 |
| - Gd64 |  | >25 | cF4 | Cu |  |
| - Gd64 |  | >36 | hP6 | Pr |  |
| - Tb65 | <220 |  | oC4 | '-Dy |  |
| '- Tb65 |  |  | hP2 | Mg | 1.580 |
| - Tb65 | >1562 |  | cI2 | W |  |
| - Tb65 |  | >3.0 | hR3 | -Sm | 4.5 x 1.60 |
| - Tb65 |  | >6.0 | hP4 | -La |  |
| - Tb65 |  | >29 | cF4 | Cu |  |
| - Tb65 |  | >32 | hP6 | Pr | 3 x 1.616 |
| - Dy66 | <86 |  | oC4 | '-Dy |  |
| '- Dy66 |  |  | hP2 | Mg | 1.573 |
| - Dy66 | >1654 |  | cI2 | W |  |
| - Dy66 |  | >5.0 | hR3 | -Sm | 4.5 x 1.606 |
| - Dy66 |  | >9.0 | hP4 | -La |  |
| - Dy66 |  | >38 | cF4 | Cu |  |
| - Ho67 |  |  | hP2 | Mg | 1.570 |
| - Ho67 | >1660 |  | cI2 | W |  |
| - Ho67 |  | >7.0 | hR3 | -Sm | 4.5 x 1.63 |
| - Ho67 |  | >13 | hP4 | -La |  |
| - Er68 |  |  | hP2 | Mg | 1.569 |
| - Er68 |  | >7.0 | hR3 | -Sm |  |
| - Er68 |  | >13 | hP4 | -La |  |
| - Tm69 |  |  | hP2 | Mg | 1.570 |
| - Tm69 | >1800 |  | cI2 | W |  |
| - Tm69 |  | >9 | hR3 | -Sm | 4.5 x 1.57 |
| - Tm69 |  | >13 | hP4 | -La |  |
| - Yb70 | <270 | or >34 | hP2 | Mg | 1.646 |
| - Yb70 |  |  | cF4 | Cu |  |
| - Yb70 | >1047 | or >3.5 | cI2 | W |  |
| - Lu71 |  |  | hP2 | Mg | 1.583 |
| - Lu71 |  | >18 | hR3 | -Sm | 4.5 x 1.52 |
| - Yb70 |  | >35 | hP4 | -La |  |
| - Hf72 |  |  | hP2 | Mg | 1.581 |
| - Hf72 | >2016 |  | cI2 | W |  |
| - Hf72 |  | >38 | hP3 | - Ti |  |
| - Hf72 |  | >71 | cI2 | W |  |
| Ta73 |  |  | cI2 | W |  |
| W74 |  |  | cI2 | W |  |
| Re75 |  |  | hP2 | Mg | 1.615 |
| Os76 |  |  | hP2 | Mg | 1.580 |
| Ir77 |  |  | cF4 | Cu |  |
| Pt78 |  |  | cF4 | Cu |  |
| Au79 |  |  | cF4 | Cu |  |
| - Hg80 | <234.3 |  | hR1 | - Hg |  |
| - Hg80 |  | >3.7 | tI2 | - Pa |  |
| - Hg80 |  | >12 | oP4 |  |  |
| - Hg80 |  | >37 | hP2 | Mg | 1.76 |
| - Tl81 |  |  | hP2 | Mg | 1.598 |
| - Tl81 | >503 |  | cI2 | W |  |
| - Tl81 |  | >3.7 | cF4 | Cu |  |
| - Pb82 |  |  | cF4 | Cu |  |
| - Pb82 |  | >13.7 | hP2 | Mg | 1.650 |
| - Pb82 |  | >109 | cI2 | W |  |
| - Bi83 |  |  | hR2 | - As | 2.609 |
| - Bi83 |  | >2.6 | mC4 | - Bi |  |
| - Bi83 |  | >3.0 | mP4 | - Sb |  |
| - Bi83 |  | >4.3 |  |  |  |
| - Bi83 |  | >9.0 | cI2 | W |  |
| - Po84 |  |  | cP1 | - Po |  |
| - Po84 | >327 |  | hR1 | - Po |  |
| Ra88 |  |  | cI2 | W |  |
| Ac89 |  |  | cF4 | Cu |  |
| - Th90 |  |  | cF4 | Cu |  |
| - Th90 | >1633 |  | cI2 | W |  |
| - Pa91 |  |  | tI2 | -Pa | 0.825 |
| - Pa91 | >1443 |  | cI2 | W |  |
| - U92 |  |  | oC4 | -U |  |
| - U92 | >941 |  | tP30 | -U |  |
| - U92 | >1049 |  | cI2 | W |  |
| - Np93 |  |  | oP8 | -Np |  |
| - Np93 | >553 |  | tP4 | -Np | 0.694 |
| - Np93 | >849 |  | cI2 | W |  |
| - Pu94 |  |  | mP16 | -Pu |  |
| - Pu94 | >388 |  | mC34 | -Pu |  |
| - Pu94 | >488 |  | oF8 | -Pu |  |
| - Pu94 | >583 |  | cF4 | Cu |  |
| '- Pu94 | >725 |  | tI2 | In | 1.342 |
| - Pu94 | >756 |  | cI2 | W |  |
| - Pu94 |  | >40.0 | hP8 |  | 1.657/2 |
| - Am95 |  |  | hP4 | -La | 2 x 1.621 |
| - Am95 | >1042 | or >5 | cF4 | Cu |  |
| - Am95 | >1350 |  | cI2 | W |  |
| - Am95 |  | >12.5 | mP4 | - Am |  |
| - Am95 |  | >15 | oC4 | - U |  |
| - Cm96 |  |  | hP4 | -La | 2 x 1.621 |
| - Cm96 | >1550 | or >23 | cF4 | Cu |  |
| - Cm96 |  | >43 | oC4 | - U |  |
| - Bk97 |  |  | hP4 | -La | 2 x 1.620 |
| - Bk97 | >1250 | or >8 | cF4 | Cu |  |
| - Bk97 |  | >25 | oC4 | - U |  |
| - Cf98 |  |  | hP4 | -La | 2 x 1.625 |
| - Cf98 | >863 | or >17 | cF4 | Cu |  |
| - Cf98 |  | >30 | aP4 | - Cf |  |
| - Cf98 |  | >41 | oC4 | - U |  |
| - Es99 |  |  | hP4 | -La |  |
| - Es99 | ? |  | cF4 | Cu |  |

Appendix B.1 - Slip and Twinning Planes in HCP Metals

Data source: F. Aldinger, in Beryllium Science and Technology (D. Webster and G. London, eds.), Vol. 1, Plenum Press, New York, p. 19 (1979).

|  | c/a | F1 Ratio | Primary Slip Plane | Secondary Slip Planes | Twinning Planes | Basal Plane Stacking Fault Energy (mJ/m2) | Prism Plane Stacking Fault Energy (mJ/m2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Co | 1.623 | 26.6 | (0001) | 1122,1011 | 1012,1121 | 26 | - |
| La | 1.6132 | 15.9 | (0001) | - | - | - | - |
| Cd | 1.886 | 2.22 | (0001) | 1122,1011, 1010 | 1012 | 15-150 | - |
| Zn | 1.856 | 1.59 | (0001) | 1122,1010, 1011 | 1012 | 15-300 | - |
| Re | 1.615 | 1.53 | (0001) | 1010,1011 | 1012,112n | - | - |
| Be | 1.568 | 1.51 | (0001) | 1010, 1122 | 1012, 112n | 173-1000 | ≥1190 |
| Mg | 1.623 | 1.49 | (0001) | 1122, 1010, 1011 | 101n | 30-60 | - |
| Tb | 1.581 |  | (0001), 1010 | - | - | - | - |
| Y | 1.571 |  | 1010 | (0001) | 1012,112n | - | - |
| Gd | 1.590 |  | 1010 | - | - | - | - |
| Ru | 1.582 |  | 1010 | - | 101n,112n | - | - |
| Os | 1.579 |  | 1010 | - | 101n | - | - |
| Hf | 1.581 |  | 1010 | (0001), 1011, 1122 | 1012,112n | - | - |
| Ti | 1.587 | 0.95 | 1010 | 1011,(0001) | 1012,112n | 300 | 145 |
| Zr | 1.593 | 0.87 | 1010 | 1011,(0001), 1122 | 1012,112n | 300 | 56 |
| Tl | 1.598 | 0.49 | 1010 | (0001) | - | - | - |

1.  F Ratio = F(hcp-bcc)/F(hcp-fcc)

2.  La has the double HCP structure.

Appendix B.2 - Elastic Moduli and Poisson's Ratios of HCP Metals

Data source: F. Aldinger, in Beryllium Science and Technology (D. Webster and G. London, eds.), Vol. 1, Plenum Press, New York, p. 10 (1979).

|  | Bulk Modulus (B) (GPa) | Shear Modulus () (GPa) | Young's Modulus (E) (GPa) | Poisson's Ratio | B/ | Ductility |
| --- | --- | --- | --- | --- | --- | --- |
| Tl | 35.6 | 5.4 | 15.3 | 0.407 | 5.02 | Good |
| Zr | 95.3 | 35.3 | 96.5 | 0.335 | 2.70 | Good |
| Cd | 62.9 | 25.1 | 66.6 | 0.324 | 2.50 | Fair |
| Ti | 103.5 | 43.4 | 114.1 | 0.316 | 2.39 | Good |
| Mg | 35.6 | 17.3 | 44.8 | 0.290 | 2.05 | Fair |
| Re | 363.5 | 181.0 | 465.7 | 0.287 | 2.01 | Fair |
| La | 28 | 14 | 37 | 0.28 | 2 | Good |
| Hf | 108.6 | 56.0 | 143.4 | 0.280 | 1.94 | Good |
| Zn | 72.4 | 44.6 | 110.9 | 0.245 | 1.62 | Poor |
| Co | 128.7 | 84.4 | 207.8 | 0.231 | 1.52 | Fair |
| Ru | 220 | 173 | 447 | .27 | 1.27 | Poor |
| Tb | 38.7 | 22 | 56 | 0.26 | 1.76 |  |
| Os | 222.0 | 234.4 | 551.6 | 0.25 | 0.95 | Poor |
| Gd | 38 | 22 | 55 | 0.26 | 1.72 |  |
| Be | 111.6 | 157.4 | 321.1 | 0.020 | 0.71 | Poor |
| Y | 15.4 | 25.5 | 49.3 | -0.034 | 0.60 | Fair |

Appendix C - Abundance of the Elements in the Earth's Crust

Data source: W.S. Fyfe, Geochemistry, Oxford University Press (1974).

| Rank | Element | Weight ppm | Summation, % |
| --- | --- | --- | --- |
| 1 | O | 455,000 | 45.50 |
| 2 | Si | 272,000 | 72.70 |
| 3 | Al | 83,000 | 81.00 |
| 4 | Fe | 62,000 | 87.20 |
| 5 | Ca | 46,600 | 91.86 |
| 6 | Mg | 27,640 | 94.62 |
| 7 | Na | 22,700 | 96.89 |
| 8 | K | 18,400 | 98.73 |
| 9 | Ti | 6320 | 99.36 |
| 10 | H | 1520 | 99.51 |
| 11 | P | 1120 | 99.63 |
| 12 | Mn | 1060 | 99.73 |
| 13 | F | 544 | 99.79 |
| 14 | Ba | 390 | 99.83 |
| 15 | Sr | 384 | 99.86 |
| 16 | S | 340 | 99.90 |
| 17 | C | 180 | 99.92 |
| 18 | Zr | 162 | 99.93 |
| 19 | V | 136 | 99.95 |
| 20 | Cl | 126 |  |
| 21 | Cr | 122 |  |
| 22 | Ni | 99 |  |
| 23 | Rb | 78 |  |
| 24 | Zn | 76 |  |
| 25 | Cu | 68 |  |
| 26 | Ce | 66 |  |
| 27 | Nd | 40 |  |
| 28 | La | 35 |  |
| 29 | Y | 31 |  |
| 30 | Co | 29 |  |
| 31 | Sc | 25 |  |
| 32 | Nb | 20 |  |
| 33 | N, Ga | 19 |  |
| 35 | Li | 18 |  |
| 36 | Pb | 13 |  |
| 37 | Pr | 9.1 |  |
| 38 | B | 9 |  |
| 39 | Th | 8.1 |  |
| 40 | Sm | 7.0 |  |
| 41 | Gd | 6.1 |  |
| 42 | Er | 3.5 |  |
| 43 | Yb | 3.1 |  |
| 44 | Hf | 2.8 |  |
| 45 | Cs | 2.6 |  |
| 46 | Br | 2.5 |  |
| 47 | U | 2.3 |  |
| 48 | Sn, Eu | 2.1 |  |
| 50 | Be | 2 |  |
| 51 | As | 1.8 |  |
| 52 | Ta | 1.7 |  |
| 53 | Ge | 1.5 |  |
| 54 | Ho | 1.3 |  |
| 55 | Mo, W, Tb | 1.2 |  |
| 58 | Tl | 0.7 |  |
| 59 | Tm | 0.5 |  |
| 60 | I | 0.46 |  |
| 61 | In | 0.24 |  |
| 62 | Sb | 0.2 |  |
| 63 | Cd | 0.16 |  |
| 64 | Ag, Hg | 0.08 |  |
| 66 | Se | 0.05 |  |
| 67 | Pd | 0.015 |  |
| 68 | Pt | 0.01 |  |
| 69 | Bi | 0.008 |  |
| 70 | Os | 0.005 |  |
| 61 | Au | 0.004 |  |
| 72 | Ir,Te | 0.001 |  |
| 74 | Re | 0.0007 |  |
| 75 | Ru,Rh | 0.0001 |  |

Appendix D - Ductility-related parameters for Common Metals

Data source: R.M. Thomson, Physical Metallurgy, Vol. III, 4th ed., p. 2261,North-Holland (1996).

| Material | Free Surface Energy, s (mJ/m2) | Shear Modulus for Slip, G (GPa) | Dislocation Burgers Vector Length1 (nm) | Unstable Stacking Fault Energy2, us (mJ/m2) | s/us (Embedded Atom Method)3 |
| --- | --- | --- | --- | --- | --- |
| FCC |  |  |  |  |  |
| Ag | 1340 | 25.6 | 0.166 | 150 | 16.1 |
| Al | 1200 | 25.1 | 0.165 | 150 | 14.6 |
| Au | 1560 | 23.7 | 0.166 | 140 | 20.1 |
| Cu | 1790 | 40.8 | 0.147 | 220 | 15.1 |
| Ir | 2950 | 198 | 0.156 | 1100 | 4.8 |
| Ni | 2270 | 74.6 | 0.144 | 390 | 10.7 |
| Pb | 610 | 7.27 | 0.201 | 53 | 21.1 |
| Pt | 2590 | 57.5 | 0.160 | 330 | 14.3 |
| BCC |  |  |  |  |  |
| Cr | 2320 | 131 | 0.250 | 2000 | 2.1 |
| Fe | 2370 | 69.3 | 0.248 | 1100 | 4.1 |
| K | 130 | 1.15 | 0.453 | 32 | 7.4 |
| Li | 530 | 3.9 | 0.302 | 73 | 13.2 |
| Mo | 2280 | 131 | 0.273 | 2200 | 1.9 |
| Na | 240 | 2.43 | 0.366 | 55 | 7.9 |
| Nb | 2570 | 46.9 | 0.286 | 830 | 5.6 |
| Ta | 2900 | 62.8 | 0.286 | 1100 | 4.7 |
| V | 2280 | 50.5 | 0.262 | 820 | 5.1 |
| W | 3070 | 160 | 0.274 | 2700 | 2.1 |

1. Burgers vector is for a perfect dislocation in BCC and for a partial dislocation in FCC.

2. Calculated from G/22 for simple cubic, with appropriate corrections for FCC, BCC lattices

3. The criteria for predicted ductility are s/us > 2.9 for FCC and s/us > 2.4 for BCC.