Borland Turbo Pascal / Turbo C BGI Driver for TSENG LABS VGA Cards

==================================================================

(C) Copyright 1990 by TSS Rolf Bartz, West Germany

Annastra�e 2b, D-5102 W�rselen, Fax: +49 2405 94986

BGIDEMO program (C) Copyright 1988 by Borland International, Inc.

Used with kind permission of Borland International, Inc.

The following files are on your disk:

TSENG16.BGI    BGI driver for TSENG LABS VGA cards, 16 color modes

TSENG256.BGI   BGI driver for TSENG LABS VGA cards, 256 color modes

BGIDEMO.EXE    Demonstration program for BGI drivers

READMEGR.DOC   German documentation

READMEUS.DOC   English documentation

Supported VGA Cards

===================

This BGI driver supports TSENG LABS EVA/800 (identical with OPTIMA VGA

Sync 800), TSENG LABS EVA/1024 (identical with OPTIMA VGA Sync 1024),

TSENG LABS MEGA-EVA/800 and MEGA-EVA/1024. Maximum available resolutions

for each card are:

Card                                      16 Colors        256 Colors

----------------------------------------  ---------------  ---------------

TSENG LABS EVA/800 (256KB)                max 800 x 600    max 320 x 200

TSENG LABS EVA/1024 (512KB)               max 1024 x 768   max 800 x 600

TSENG LABS MEGA-EVA/800 (256KB)           max 800 x 600    max 320 x 200

TSENG LABS MEGA-EVA/1024 (1MB)            max 1024 x 768   max 1024 x 768

TSENG16.BGI BGI Driver

======================

This BGI driver supports the standard and extended 16 color modes of the

TSENG LABS VGA cards. The following modes are available:

Mode  Resolution             Cards

----  ---------------------  -----------------------------------------

1    320 x 200  16 colors  EGA

2    640 x 200  16 colors  EGA

3    640 x 350  16 colors  EGA

4    640 x 480  16 colors  VGA

5    640 x 480  16 colors  all supported cards

6    720 x 512  16 colors  none

7    800 x 600  16 colors  all supported cards

8   1024 x 768  16 colors  EVA/1024

9   1024 x 768  16 colors  MEGA-EVA/1024

Since the modes 0 to 3 are standard IBM modes, they work with every standard

EGA/VGA compatible card, including the TSENG LABS VGA cards (Mode 4 with VGA

only). This eliminates the need to use Borlands EGAVGA driver. Use this driver

instead. All other modes work only with the cards given in the Cards column.

Although mode 6 is documented by TSENG LABS, it does not work with any card.

TSENG256.BGI BGI Driver

=======================

This BGI driver supports the standard and extended 256 color modes of the

TSENG LABS VGA cards. The following modes are available:

Mode  Resolution              Cards

----  ----------------------  -----------------------------------------

1    320 x 200  256 colors  VGA

2    640 x 350  256 colors  EVA/1024

3    640 x 350  256 colors  MEGA-EVA/1024

4    640 x 400  256 colors  MEGA-EVA/1024

5    640 x 480  256 colors  EVA/1024

6    640 x 480  256 colors  MEGA-EVA/1024

7    720 x 512  256 colors  none

8    800 x 600  256 colors  EVA/1024

9    800 x 600  256 colors  MEGA-EVA/1024

10   1024 x 768  256 colors  MEGA-EVA/1024

Since the mode 0 is a standard IBM mode, it works with every standard VGA

compatible card (including the TSENG LABS VGA cards). All other modes work only

with the cards given in the Cards column.

Although mode 7 is documented by TSENG LABS, it does not work with any card.

How to include the drivers in Turbo Pascal

==========================================

For Turbo Pascal, you must include the following code in your program.

Supported is Turbo Pascal version 5.x and version 6.0.

var GraphDriver:         integer;  (* global variable for use with InitGraph *)

GraphMode:           integer;  (* global variable for use with InitGraph *)

TSENG16DriverMode:   integer;  (* global variable to hold TSENG16 mode *)

TSENG256DriverMode:  integer;  (* global variable to hold TSENG256 mode *)

(* AutoDetect function for TSENG16 driver. Each driver must have its own *)

(* AutoDetect function. It is called by InitGraph and must return the mode *)

(* number to be used if this driver shall get activated or grError if this *)

(* driver is not going to be used. *)

(*$F+*) function AutoDetectTSENG16: integer (*$F-*);

begin

AutoDetectTSENG16 := TSENG16DriverMode

end;

(* AutoDetect function for TSENG256 driver. *)

(*$F+*) function AutoDetectTSENG256: integer (*$F-*);

begin

AutoDetectTSENG256 := TSENG256DriverMode

end;

procedure InitializeGraphics;

var DriverNumber, ErrorCode: integer;

begin

(* ... other code ... *)

(* Add TSENG16 driver to driver table and assign AutoDetect function. *)

(* You must NOT specify a path here, this is done later with initgraph. *)

DriverNumber := InstallUserDriver ('TSENG16', @AutoDetectTSENG16);

if GraphResult <> grOK then begin

WriteLn ('Error installing BGI driver TSENG16.');

Halt (1)

end;

(* Add TSENG256 driver to driver table and assign AutoDetect function. *)

(* You must NOT specify a path here, this is done later with initgraph. *)

DriverNumber := InstallUserDriver ('TSENG256', @AutoDetectTSENG256);

if GraphResult <> grOK then begin

WriteLn ('Error installing BGI driver TSENG256.');

Halt (1)

end;

(* ... other code ... *)

(* Init graphics mode, for example TSENG16 mode 7, 800x600, 16 colors *)

GraphDriver := Detect;             (* request autodetection *)

TSENG16DriverMode := 7;            (* enable TSENG16 driver in mode 7 *)

TSENG256DriverMode := grError;     (* disable all other drivers *)

InitGraph (GraphDriver, GraphMode, '');

ErrorCode := GraphResult;

if ErrorCode <> grOK then begin

WriteLn ('Graphics error: ', GraphErrorMsg (ErrorCode));

Halt (1)

end;

(* ... other code ... *)

end;

How to include the drivers in Turbo C

=====================================

For Turbo C, you must include the following code in your program.

Supported is Turbo C version 2.0 and Turbo C++ version 1.0.

int GraphDriver;          /* global variable for use with InitGraph */

int GraphMode;  	  /* global variable for use with InitGraph */

int TSENG16DriverMode;    /* global variable to hold TSENG16 mode */

int TSENG256DriverMode;   /* global variable to hold TSENG16 mode */

/* AutoDetect function for TSENG16 driver. Each driver must have its own */

/* AutoDetect function. It is called by InitGraph and must return the mode */

/* number to be used if this driver shall get activated or grError if this */

/* driver is not going to be used. */

int huge AutoDetectTSENG16 (void)

{

return (TSENG16DriverMode);

}

/* AutoDetect function for TSENG256 driver. */

int huge AutoDetectTSENG256 (void)

{

return (TSENG256DriverMode);

}

void InitializeGraphics (void)

{

int DriverNumber;

int ErrorCode;

/* ... other code ... */

/* Add TSENG16 driver to driver table and assign AutoDetect function. */

/* You must NOT specify a path here, this is done later with initgraph. */

DriverNumber = installuserdriver ("TSENG16", AutoDetectTSENG16);

if (graphresult () != grOk) {

printf ("Error installing BGI driver TSENG16.\n");

exit (1);

}

/* Add TSENG256 driver to driver table and assign AutoDetect function. */

/* You must NOT specify a path here, this is done later with initgraph. */

DriverNumber = installuserdriver ("TSENG256", AutoDetectTSENG256);

if (graphresult () != grOk) {

printf ("Error installing BGI driver TSENG256.\n");

exit (1);

}

/* ... other code ... */

/* Init graphics mode, for example TSENG16 mode 7, 800x600, 16 colors */

GraphDriver = DETECT;              /* request autodetection */

TSENG16DriverMode = 7;             /* enable TSENG16 driver in mode 7 */

TSENG256DriverMode = grError;      /* disable all other drivers */

initgraph (&GraphDriver, &GraphMode, "");

ErrorCode = graphresult ();

if (ErrorCode != grOk) {

printf ("Graphics Error: %s\n", grapherrormsg (ErrorCode));

exit (1);

}

/* ... other code ... */

}

BGIDEMO BGI driver sample program

=================================

This is a modified version of the BGIDEMO program shipped by Borland to

demonstrate the features of their BGI drivers. It is adapted to support

installable drivers. The usage is:

BGIDEMO Driver [Mode]

where Driver is the name of the driver (TSENG16 or TSENG256), and Mode is the

requested mode number. If you omit Mode, you will get a list of all available

modes. This feature only works with our BGI drivers.

Enhancements

============

All our BGI drivers have an "Info Mode": mode number 0. If you use the normal

InitGraph procedure to start up the BGI driver in mode 0, the graphics system

will be enabled, but there will be NO video mode switching. So you can use

the GetModeRange procedure and the GetModeName function to get information

about the supported video modes without actually switching to any mode.

The special function GetModeName (0) returns the name of the driver, for

example "BGI Driver TSENG LABS V2.00, 16 Colors".

Restrictions

============

Due to some bugs in the GRAPH.TPU unit, the following functions do not work

properly:

- The function GetDriverName returns an empty string when used with installable

drivers. This does not occur with Turbo C. The function GetModeName is OK.

- The procedure InitGraph does not chain error codes that are generated by the

BGI driver back to the application code. The GraphResult function always

returns OK even if an error occurred inside the BGI driver. So it is not

possible to check whether the requested graphics mode really is used.

If you know what BIOS video mode you request, you can use the function

GetVideoMode below to check the BIOS video mode after the call to InitGraph.

function GetVideoMode: byte;

var Reg: Registers; (* uses DOS *)

begin

Reg.AH := $0F;            (* Video BIOS function $0F *)

Intr ($10, Reg);          (* call Video BIOS *)

GetVideoMode := Reg.AL;   (* AL now contains current video mode *)

end;

- The Borland Graphics Interface is laid out primarily for 16 colors. The

type PaletteType can only be used for 16 color palettes. The functions

GetPalette, SetPalette and SetAllPalette only affect the old 16 color EGA

palette (also present in the 16 color VGA modes). Do not use them with

the 256 color modes, if you do not know exactly what you do.

- The only possible operation to modify the extended VGA 256 color palette is

SetRGBPalette, which turns out to be very slow if you want to set the whole

palette. Try this procedure instead:

type VGAPaletteType = array [0..255] of record R, G, B: byte end;

procedure SetAllVGAPalette (var Palette: VGAPaletteType);

var Reg: Registers; (* uses DOS *)

begin

Reg.AX := $1012;          (* VGA BIOS function $10, subfunction $12 *)

Reg.BX := 0;              (* first palette entry to set *)

Reg.CX := 256;            (* number of palette entries to set *)

Reg.ES := Seg (Palette);  (* segment of palette data *)

Reg.DX := Ofs (Palette);  (* offset of palette data *)

Intr ($10, Reg)           (* call VGA BIOS *)

end;

- Because the overscan color sometimes causes unpredictable results on EGA

and VGA cards, the function SetBkColor, which was used mainly with the

CGA adapter, is disabled in this BGI drivers. Please avoid using this

function, because the meaning of SetBkColor may change in future BGI

releases. If you really want to set the overscan color, use the following

procedure:

procedure SetOverScanColor (Color: integer);

var Reg: Registers; (* uses DOS *)

begin

Reg.AX := $1001;          (* VGA BIOS function $10, subfunction $01 *)

Reg.BH := Color;          (* overscan color to set *)

Intr ($10, Reg)           (* call VGA BIOS *)

end;

The overscan color provided with the SetAllPalette procedure is also ignored.