Program NNET

Belongs to the STATMOD package

Generation of synthetic pattern sets from the pattern source defined by the statistical model. Display of the two-dimensional measurement space with class-conditional distributions, class boundaries and class regions. Calculation of classifiers according to recursive learning rules: recursive complete quadratic polynomial regression classifier (.qmk) and multilayer perceptron regression classifier (.nnt). Performance evaluation of these classifiers and display of performance diagrams.

Main 	Exit only from this level

| 1 | ->PLOT | To submenu - PLOT |
| --- | --- | --- |
| 2 | ->COOR | To submenu - COOR |
| 3 | ->STMD | To submenu - STMOD |
| 4 | ->SMPL | To submenu - SAMPLE |
| 5 | -> REC | To submenu - RECURS |
| 6 | -> CLF | To submenu - CLF |
| 7 | ->PARA | To submenu - PARAM |
| 8 | -> EDI | Editor call |
| 9 | -> DOS | DOS call |
| 10 | IMGEXT | To graphic screen and from there exit with Esc |

Plot		Display of the two-dimensional measurement space with class-				conditional distributions and class boundaries calculated by 			Bayes rule from the normal distributions. Start of the recursive 			learning procedure running over the determined sample set.

| 1 | COOR | Draw coordinate system |
| --- | --- | --- |
| 2 | AXES | Draw class-conditional principal axes |
| 3 | ELLIPS | Draw class-conditional ellipses |
| 4 | BORDER | Draw class boundaries |
| 5 | RLV_BD | Toggle RLV_BD « ALL_BD RLV_BD: only active sections of the class boundaries ALL_BD: complete boundaries are drawn |
| 6 | NO_PRO | Toggle NO_PRO « PRO PRO: activates the protocol function during recursive learning NO_PRO: protocol function off |
| 7 | MODE_0 | Choice of MODE_0 « MD_CLF « MD_NNT MODE_0: no classifier activated MD_CLF: activates polynomial regression classifier MD_NNT: activates multilayer perceptron regression classifier |
| 8 | START | Start process running over the whole sample set |
| 9 | CLEAR | Clear graphic screen |
| 10 | MAIN<- | Return to menu MAIN |

Coor   	Coordinate system

| 1 | PLOT | Draw coordinate system |
| --- | --- | --- |
| 2 | EDIT | Edit coordinate system |
| 3 | LD_COO | Load coordinate system from file |
| 4 | SV_COO | Save coordinate system to file |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 | IMAGE | To graphic screen |
| 9 | CLEAR | Clear graphic screen |
| 10 | MAIN<- | Return to menu MAIN |

StMod  	Statistical Models

Allows editing of model parameters and changing of a-priori-			probabilities and colors for the display of the different classes

| 1 | ED_APR | Edit apriori probabilities |
| --- | --- | --- |
| 2 | ED_COL | Edit class specific colors |
| 3 | LD_MOD | Load statistical model from file |
| 4 | SV_MOD | Save statistical model to file |
| 5 |  |  |
| 6 |  |  |
| 7 | -> EDI | Editor call |
| 8 | R_SEED | Set default value random generator seed variable |
| 9 | R_INIT | Initialize random generator |
| 10 | MAIN<- | Return to menu MAIN |

Sample 	Determines details on sample sets

| 1 | POIPLT | Choice of POIPLT « SYMPLT « NOPLT Each sample of the sample set is shown POIPLT: as a point SYMPLT: as a character representing the class label NOPLT: not shown in the measurement space |
| --- | --- | --- |
| 2 | NUMBER | Total sample size to be processed |
| 3 | RDSMPL | Filename sample set input |
| 4 | WRSMPL | Filename sample set output |
| 5 | NOCYKL | Toggle of CYKL « NOCYKL Effective only if the sample set is read from file CYKL: in case of unsufficient number of samples contained in the file the file is cyclically reopened and samples read until the required number is reached NOCYKL: process stops at end of file |
| 6 | MODEL | Toggle MODEL « SMPSET MODEL: sample set read from random generator controlled by the statistical model SMPSET: sample read from file |
| 7 |  |  |
| 8 | NO_CNT | Toggel CNT « NO_CNT During any process running over the whole sample set the individual samples are on a per class level counted (if at the same time any classifier is active, together with the classification errors) CNT: counting on NO_CNT: no counting After completion the result is shown in an extra window |
| 9 | SH_CNT | Show the last counting result |
| 10 | MAIN<- | Return to menu MAIN |

Recurs   	Entrance menu for recursive classifier adaptation. The protocol file 			keeps trace of the learning procedure. The learning rule can be 			switched off for measuring the same type of protocol with a 				fixed classifier

| 1 | -> NNT | To submenu - NNTMK |
| --- | --- | --- |
| 2 | -> CLF | To submenu - CLFMK |
| 3 |  |  |
| 4 | STPPRO | Recording stepwidth for the protocol file |
| 5 | SV_PRO | Pathname for the protocol file |
| 6 | NOLEAR | Toggle NOLEAR « LEAR LEAR: the present classifer is updated for each input vector -- singular learning mode recursive learning NOLEAR: the present classifier remains unchanged |
| 7 | SH_NOT | Toggel SH_NOT « SH_PRO SH_PRO: displays the protocol data in the graphic screen during recursive learning SH_NOT: display off |
| 8 |  |  |
| 9 |  |  |
| 10 | MAIN<- | Return to menu MAIN |

Clf    	Classifier menu, loading and saving of polynomial classifiers. 				Branches to several submenus dealing with classifier 					computation and evaluation of classifier performance

| 1 |  |  |
| --- | --- | --- |
| 2 |  |  |
| 3 | LD_CLF | Load classifier from file |
| 4 | SV_CLF | Save classifier to file |
| 5 | ->PLOT | To submenu CLFPLT |
| 6 | -> REC | To submenu RECOGN |
| 7 | LD_STA | Load classifier evaluation data .sta representing mean number of alternatives and residual error rate |
| 8 | SV_STA | Save classifier evaluation data .sta representing mean number of alternatives and residual error rate |
| 9 | ->STPL | To submenu STAPLT |
| 10 | MAIN<- | Return to menu MAIN |

NntMk    	Multilayer perceptron training -- backpropagation

| 1 | CREATE | Generate a new multilayer perceptron classifier |
| --- | --- | --- |
| 2 | LD_NNT | Load classifier from file |
| 3 | SV_NNT | Save classifier to file |
| 4 | ->PLOT | To submenu NNTPLT |
| 5 |  |  |
| 6 | ALPMIN | Lower limit for the learning factor in case of ALPVAR |
| 7 | ALPCON | Toggel ALPCON « ALPVAR ALPCON: learning factor constant during learning ALPVAR: learning factor reduced in steps during learning |
| 8 | ALPHA | Learning factor |
| 9 | INIWGH | Limits of the interval from which the initial weights are taken |
| 10 | REC <- | Return to menu RECURS |

ClfMk  	Recursive training of the complete quadratic polynomial classifier

| 1 | CREATE | Generate a new polynomial regression classifier -- all weights zero |
| --- | --- | --- |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 | -> EDI | Editor call |
| 7 |  |  |
| 8 | ALPHA | Learning factor |
| 9 |  |  |
| 10 | REC <- | Return to menu RECURS |

NntPlt 	Display of the two-dimensional measurement space with class				regions as determined by the multilayer perceptron classifier 				(.nnt)

| 1 | COOR | Draw coordinate system |
| --- | --- | --- |
| 2 | REGION | Plot class regions according to best choice classifier result |
| 3 | BORDER | Plot class boundaries |
| 4 | FIRSTL | Draw first layer boundaries |
| 5 | ALL | Choice of ALL « NODE_1 « NODE_2 « NODE_i depending on the number of hidden nodes in the first perceptron layer NODE_i: activation of button FIRSTL displays the borderline represented by (node_i)-function = 0.5 ALLE: displays the whole set of first layer boundaries |
| 6 | LCOLOR | Line color |
| 7 | LSTYLE | Line style |
| 8 | IMAGE | To graphic screen |
| 9 | CLEAR | Clear graphic screen |
| 10 | NNT <- | Return to menu NNT |

ClfPlt 	Display of the two-dimensional measurement space with class				regions, discriminant functions and class boundaries as 				determined by the polynomial regression classifier (.qmk)

| 1 | COOR | Draw coordinate system |
| --- | --- | --- |
| 2 | REGION | Plot class regions according to best choice classifier result |
| 3 | D_FCT | Draw discriminant functions as contourplots |
| 4 | BORDER | Draw class boundaries |
| 5 | RLV_BD | Toggle RLV_BD « ALL_BD RLV_BD: only active sections of the class boundaries ALL_BD: complete boundaries are drawn |
| 6 |  |  |
| 7 |  |  |
| 8 | IMAGE | To graphic screen |
| 9 | CLEAR | Clear graphic screen |
| 10 | CLF <- | Return to menu CLF |

Recogn 		Parameter Setting for classification

| 1 | OUTLTH | Outlier-threshold used for classifiers with a simplified confidence mapping technique, where the 'eigen'- and 'fremd'-histograms are approximated by Normal distributions with a certain mean and a certain standard deviation. |
| --- | --- | --- |
| 2 | CONFTH | Reject threshold requiring a minimum value of the discriminant function, effective for region plot in menu CLFPLT and for counting classification errors |
| 3 | RADQTH | Reject threshold allowing only a maximum square distance between the discriminant vector in decision space and the nearest target point, effective for region plot in menu CLFPLT and for counting classification errors (.qmk) |
| 4 | WRDVKT | Filename set of discriminant vectors -- output |
| 5 |  |  |
| 6 | NODSPL | Toggle DSPL «NODSPL DSPL: displays the classification result in an extra window NODSPL: display of the classification result off |
| 7 | NOSTEP | Toggle STEP «NOSTEP STEP: program execution stops after every single sample NOSTEP: singlestep off |
| 8 | NOSTAT | Toggle STAT «NOSTAT STAT: collection of statistical data for classifier performance evaluation NOSTAT: collection of statistical data off provided that a classifier is present, computed or loaded |
| 9 | SELECT | When running a classifier selected measurement and discriminant vectors can be written on .sti rsp. .dvk files. The selection is controlled by the classification result according to the selection code asked here |
| 10 | CLF <- | Return to menu CLF |

StaPlt 		Display of functions concerning mean number of alternatives and 			residual error rate

Use appropriate coordinate system.

| 1 | COOR | Draw coordinate system |
| --- | --- | --- |
| 2 | LD_COO | Load coordinate system from file |
| 3 | EPS | Draw residual error rate as function of the confidence threshold |
| 4 | ALT | Draw mean number of alternatives as function of the confidence threshold |
| 5 | EPSALT | Draw performance diagram: residual error rate as function of the mean number of alternatives |
| 6 | LCOLOR | Line color |
| 7 | LSTYLE | Line style |
| 8 | IMAGE | To graphic screen |
| 9 | CLEAR | Clear graphic screen |
| 10 | CLF <- | Return to menu CLF |

Param  		general parameter settings and writing on the graphic screen

| 1 | ->PATH | To submenu PATHS |
| --- | --- | --- |
| 2 | ->COLS | To submenu COLORS |
| 3 | WCOLOR | Writing color for directly writing on graphic screen |
| 4 | WRTIMG | Direct writing on graphic screen Exit with 'Esc'-key |
| 5 |  |  |
| 6 | SELECT | Selection of a subset of classes 'ALLE' takes all, otherwise give desired selection |
| 7 | HYSTEP | Stepwidth for drawing hyperbolas |
| 8 |  |  |
| 9 | BLOCK | Block size for region plot |
| 10 | MAIN<- | Return to menu MAIN |

Paths  		Path settings

| 1 |  |  |
| --- | --- | --- |
| 2 | EDIT | Edit pathname settings |
| 3 | LD_PFD | Load pathnames from file |
| 4 | SV_PFD | Save pathnames to file |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 | PARA<- | Return to menu PARAM |

Colors  		Parameter settings for colors and style

| 1 |  |  |
| --- | --- | --- |
| 2 | EDIT | Edit parameter settings |
| 3 | LD_FRB | Load parameter settings from file |
| 4 | SV_FRB | Save parameter settings from file |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 | PARA<- | Return to menu PARAM |