;dotpp.asm  ASM Code with parallel instructions for fixed-point

MVK   .S1	200, A1	;count into A1

||		ZERO	.L1	A7		;init A7 for accum

LOOP		LDH	.D1   *A4++,A2	;A2=16-bit data pointed by A4

||		LDH	.D2   *B4++,B2	;B2=16-bit data pointed by B4

SUB	.S1	A1,1,A1	;decrement count

[A1]	B	.S1 	LOOP		;branch to LOOP (after ADD)

NOP		2		;delay slots for LDH and B

MPY	.M1x  A2,B2,A6	;product in A6

NOP				;1 delay slot for MPY

ADD	.L1	A6,A7,A7	;accum in A7,then branch

;branch occurs here